Dynamic Host Configuration Protocol(DHCP) Configuration

For router

enable

conf t

int fa0/0

Ip address 192.168.10.1 255.255.255.0

no shutdown

exit

ip dhcp pool IP10

net 192.168.10.0 255.255.255.0

default 192.168.10.1

exit

ip dhcp exc 192.168.10.1 192.168.10.10

exit

copy run start